Technical Specifications Document

Project Name: Perfume Filling Production Line

# 1.  Project Overview & Objectives

## 1.1. Problem Statement

Traditional filling processes in small establishments rely on manual labor, resulting in material waste (precious perfumes), inaccurate filling volumes, reduced productivity, and health risks from prolonged inhalation of volatile alcohol.

## 1.2. Project Objectives

The aimed project should be designed to achieve the following specifications:

- Full Automation: Designing a system that imitates industrial production lines to handle packages, from feeding to filling without human intervention. The bottles have size range 10–100 ml glass/plastic bottles (the common perfume sizes).

- Economic Feasibility: Designing a low-cost system, simple and effective and can fill        10-30 bottles per minuets.

- High Accuracy: Achieving high repeatability in the volume of filled liquid, the error does not exceed ± (0.5-1) % of target.

- The process is fully observed, and filling time can be varied easily via HMI as touchscreen or PC.

## 1.3. Project Scope

The project is limited to the design, simulation, and implementation of a conveyor belt, a contactless packages detection system, simple and accurate actuators for filling unit, with an emergency protection and shutdown system. The project does not include the packaging or capping stages.

# 2. Functional Specifications

## 2.1. System Modes

- Manual Mode: Allows the operator to inspect each component individually (operating the conveyor belt or filling actuator separately) for maintenance and calibration purposes.

- Automatic Mode: The system operates in a continuous, closed-loop automatic sequence.

## 2.2. Sequence of Operations

- Start: The operator presses the Start button, and the conveyor belt begins moving to transport the empty packages.

- Detection and Positioning: The contactless sensor continuously scans the distance. When a package passes and the measured distance falls below the critical threshold, an immediate command is issued to stop the conveyor belt motor.

- Filling: The system waits for 0.5 seconds to ensure the package is completely stable, then the filling actuator is activated.

- Time Control: The filling process continues to operate for the duration of the pre-programmed timer, which is precisely calculated based on the size of the target package.

- Departure and New Cycle: After the timer expires, the filling actuator stops, and the conveyor belt restarts to remove the filled package and retrieve a new one. A blind timer is activated to prevent the sensor from registering the departing package as a new one.

# Hardware Requirements

The selected hardware components must meet the following industrial and laboratory specifications:

## 3.1. Control process

Control process should consist of two parts, control unit and HMI. Control unit can be on of those two options:

- Option1: (Industrial): A programmable logic controller (PLC) (such as the Siemens S7-1200 CPU 1211C) with at least 6 digital inputs and 4 digital outputs, and a signal conversion module for sensor compatibility.

- Option2: (Academic/Laboratory): A microcontroller development board (such as the Arduino Uno or ESP32) with a processing frequency of at least 16MHz and sufficient flash memory for storing the programming code and timing libraries.

HMI part: The user can observe the filling process via an appropriate interface, with ability to varying filling time.

## 3.2. Sensors

- Sensor Type: A contactless sensor that does not emit radiation that could have an environmental impact or affect the products.

- Measuring angle: Less than 15° to ensure no wave dispersion and accurate measurement of the package edges.

- Effective measuring range: 2cm to 20cm (the average field of the filling line).

## 3.3. Filling and Agitating System (Actuators)

- Volumetric (time/pump displacement) or gravimetric (weight-based) with diving nozzles to minimize foaming/volatilization.

- The used actuator should be made of solvent and alcohol-resistant materials (avoid highly corrosive commercial plastics), with isolated driver.

- Belt Conveyor Motor: A 12V DC motor with a gearbox to ensure high torque and stable low speed (e.g., 30 to 60 RPM).

- Power: 220V AC / 24V DC control; compressed air for pneumatics (if used). The used power supply should be able to provide accurate and stable voltage to the actuator.

# 4. Software and Control Logic Specifications

- The filling time variable must be easily programmatically adjustable to accommodate different package sizes without rewriting the entire code via an appropriate interface ensure the full observation of the filling process.

- A debouncing/filtering condition must be programmed into the detecting sensor readings. The conveyor will only stop if the low distance reading stabilizes for three consecutive readings (to avoid false signals caused by ambient noise or mechanical vibration of the conveyor).

# 5. Occupational Safety and Industrial Protection

- Emergency Stop Button: A red physical button with a latching mechanism, directly connected to the main power supply line for the actuators (pump and motor). Pressing it immediately and completely disconnects the power supply, protecting the operator and machine in case of bottle jamming or uncontrolled liquid flow.

- Electrical Protection: an electrical fuse must be installed on the main line to prevent short circuits caused by the accidental introduction of alcohol into the electronic circuits.

- Physical Separation of Panels: The perfume tank and filling nozzles are mechanically isolated on one side, while the electrical and electronic panel is isolated and protected on the other side by an insulated plastic box (at least IP54 protection).

- Guards, low-level tank sensor, overflow prevention should be used.

- Materials: SS316L for liquid-contact parts; food-grade seals.

- Environment: Indoor, 15–30°C, suitable for flammable vapors (explosion-proof considerations).

# Testing and Acceptance Criteria:

The project is considered successful when the following conditions are met:

- The sensor successfully detects (10-30) consecutive packages per minute (of different colors and transparency levels) without any positioning error, prototype scalable to industrial rates.

- The filling volume error does not exceed ± (0.5-1) % of target.

- The emergency button responds immediately and the system safely restarts from the same point after unlocking.

- Complete documentation include circuits diagram, programs, and electrical specifications of used components.